# TCS MICROPUMPS: D260BLX (RANGE)

(Patents Pending, Quality Assured ISO 9001, RoHS compliant, IP41 protection rating)

## INTRODUCTION

The TCS D series Micropumps are high-quality self priming miniature pumps for gases. They are highly efficient, small and lightweight. The solid construction and wide temperature tolerance, enable them to perform reliably even in hostile environments. Your Micropump can be quickly and easily installed into the smallest spaces, in a vast range of laboratory, prototype and production equipment.

## ELECTRICAL CONNECTION

CAUTION: REVERSED OR INCORRECT CONNECTION WILL PERMANENTLY DAMAGE THE ELECTRONICS IN YOUR PUMP!

Voltage: 3.0 to 6.0 Volts dc (Maximum 0.5Amps)

The simplest connection is Red, Green and Grey together to +ve (Positive) supply, Black to –ve (Negative) of supply

| Wire Colour | Description | Connection | Status |
| --- | --- | --- | --- |
| Black | GRD The negative connection | -Ve | Required |
| Grey | VM Motor Power Supply | +Ve 3.0~6.0 | Required |
| Red | Vcc Onboard Controller Supply Circuit | +Ve 3.0~6.0 | Required |
| Green | CW/CCW Motor Rotation | +Ve 3.0~6.0 | Required |
| Yellow | RPM Signal Wire | GND Vcc 4 pulse / revolution square wave signal | Optional |

Your Micropump can be connected to batteries, a DC power source of up to 6V or a TCS power supply TCS-600 (see accessories page on website).

Pumping rate can be adjusted by varying input voltages between 3.0v and 6.0v dc. Further fine tuning can be made by restricting the outlet of your pump using a fixed jet restriction or flexible tubing and an adjustable clamp. The pump is quite happy to run even with the outlet completely blocked with gases.

## Materials

Housing:	Stainless Steel 316 (D260) 
Tubing Connectors:	Stainless Steel 316 (D260) 
Seals:	(Viton + Viton), (EPDM + EPDM), (Silicone + Nitrile), (Silicone + Viton)

TUBING

D200 series:	2.4mm (3/32”) bore flexible tubing

## Notes on Operation

All ‘D series’ pumps are self priming by design. Simply connect the pump into your flow circuit, noting the flow direction shown below, and begin pumping.

D260BLX

| Performance | Input Voltage | Power Usage | Max Pressure | Max Free Flow |
| --- | --- | --- | --- | --- |
|  | (v) | (w) | (psi) | (mlpm) |
| D260BLX - AIR | 3.0 - 6.0 | 0.1 - 0.6 | 11 | 550 |

Please note that figures above may vary depending on elastomers featured in pump.